MICROCONTROLLER CODE

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

#include <WiFi.h>

#include <ThingSpeak.h>

// ===== WIFI =====

const char* ssid = "project";

const char* password = "12345678";

// ===== THINGSPEAK =====

unsigned long channelID = 3275816;

const char* writeAPIKey = "M7T6TW8T4ITKMMGL";

WiFiClient client;

// ===== LCD =====

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ===== DHT =====

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

// ===== GAS SENSOR =====

#define GAS_PIN 34

int gasMin = 0;

int gasMax = 4095;

// ===== ALERT OUTPUT =====

#define ALERT_PIN 26   // buzzer / LED / relay

void setup() {

Serial.begin(115200);

lcd.init();

lcd.backlight();

dht.begin();

pinMode(ALERT_PIN, OUTPUT);

digitalWrite(ALERT_PIN, LOW);

// ===== WIFI CONNECT =====

lcd.print("Connecting WiFi");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

lcd.clear();

lcd.print("WiFi Connected");

delay(1500);

ThingSpeak.begin(client);

}

void loop() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

int gasValue = analogRead(GAS_PIN);

// Convert gas to %

int gasPercent = map(gasValue, gasMin, gasMax, 0, 100);

gasPercent = constrain(gasPercent, 0, 100);

if (isnan(humidity) || isnan(temperature)) {

lcd.clear();

lcd.print("Sensor Error");

return;

}

// ===== GAS ALERT + LCD MESSAGE =====

String gasStatus = "";

if (gasPercent <= 35) {

digitalWrite(ALERT_PIN, LOW);

gasStatus = "AM:" + String(gasPercent) + "%";

}

else if (gasPercent > 35 && gasPercent <= 50) {

// Normal → slow beep

digitalWrite(ALERT_PIN, HIGH);

delay(100);

digitalWrite(ALERT_PIN, LOW);

delay(1000);

gasStatus = "AMMONIA NORMAL";

}

else if (gasPercent > 50 && gasPercent <= 75) {

// Medium → fast beep

digitalWrite(ALERT_PIN, HIGH);

delay(300);

digitalWrite(ALERT_PIN, LOW);

delay(500);

gasStatus = "AMMONIA MEDIUM";

}

else {

// High → continuous ON

digitalWrite(ALERT_PIN, HIGH);

gasStatus = "AMMONIA HIGH";

}

// ===== LCD DISPLAY =====

lcd.clear();

lcd.setCursor(0,0);

lcd.print("T:");

lcd.print(temperature);

lcd.print(" H:");

lcd.print(gasStatus);

// ===== SEND TO THINGSPEAK =====

ThingSpeak.setField(1, temperature);

ThingSpeak.setField(2, humidity);

ThingSpeak.setField(3, gasPercent);

int status = ThingSpeak.writeFields(channelID, writeAPIKey);

if(status == 200){

Serial.println("Data sent");

} else {

Serial.println("Upload Failed");

}

delay(1500); // ThingSpeak minimum 15 seconds

}